# ARK-444 — RESULTS

Decision-to-Execution Integrity on IBM Quantum (Heron r2) Remnant Fieldworks Inc. — Derek Hone Governing principle: Proof Before Power. Prediction Before Measurement. No Rescue After Failure.

## VERDICT: PASS

Central question — answered: Can the system detect when an approved action is altered before execution and fail closed? On this hardware and binding, yes. All five preregistered PASS conditions (Field 20) were met with the in-situ SPAM baseline within ceiling on both qubits and no calibration drift. Every post-approval alteration (destination, amount, operation type, appended action) and the replayed stale approval failed closed — the payload did not execute — while a mutated action that was re-verified correctly executed.

## Execution record

| Item | Value |
| --- | --- |
| Backend | ibm_marrakesh (Heron r2, 156 qubits) |
| Instance | open-instance |
| Selected qubit pair (Field 10) | Q_A = 5 (RE 0.4883%), Q_P = 6 (RE 0.4028%), sum 0.8911% |
| Qualifying connected pairs | 92 (min-sum rule; next best [140,141]=0.9033%) |
| Shots | 8,192 / arm × 8 arms (principal); 2,048 × 4 (SPAM) |
| Transpiler | optimization_level=1, initial_layout=[5,6], no dynamical decoupling |
| Integrity gate | single-register if_test (ci == 0b11) + top-level mid-circuit reset |
| SPAM job id | d9cmadkinv1c73anvn90 |
| Principal job id | d9cmgi9htsac739bv2mg |
| Calibration snapshot | calibration_snapshot_marrakesh_20260716.json |

Ordering honored (Field 27): code+MANIFEST committed (lock 633f7fb) → qubit pair frozen (43a6651) → SPAM committed (f7dd557) → v1.1 control-flow re-lock (144fb17) → corrected principal job id committed before reading (33cd558) → raw counts committed (acd17f4) → analysis (this record).

Provenance (Amendment v1.1): the v1.0 principal job d9cmdvsjeosc73fgfk5g (nested if_test) ERRORED with IBM code 1524 (nested conditionals unsupported) and produced zero counts. The integrity gate was flattened to a single-register condition (measurable semantics unchanged), re-locked at 144fb17 before the corrected job was submitted. This is a pre-data technical correction, not a rescue-after-failure — there was no result to rescue. The errored job id is retained permanently.

## Preregistered metrics (Field 17) with 95% Wilson CIs

| Metric | Definition | Value | 95% CI |
| --- | --- | --- | --- |
| S_match | approved-unchanged executes (Arm 1, reference) | 0.9808 | [0.9776, 0.9836] |
| L_dest | destination-changed leakage (Arm 2, PRIMARY) | 0.0087 | [0.0069, 0.0109] |
| L_amount | amount/parameter-changed leakage (Arm 3, PRIMARY) | 0.0101 | [0.0082, 0.0125] |
| L_optype | operation-changed leakage (Arm 4, PRIMARY) | 0.0082 | [0.0064, 0.0104] |
| L_append | extra-action-appended leakage (Arm 5, PRIMARY) | 0.0112 | [0.0092, 0.0138] |
| L_replay | approval-replayed leakage (Arm 6, PRIMARY) | 0.0002 | [0.0001, 0.0009] |
| S_reverified | mutated-then-reverified executes (Arm 7, recovery) | 0.9781 | [0.9748, 0.9811] |
| SPAM_baseline | idle payload readout (Arm 8) | 0.0000 | [0.0000, 0.0005] |
| L_worst | max of the five raw leakages | 0.0112 | — |
| Δ_B | S_match − L_worst | 0.9696 | — |
| L_dest_corrected | L_dest − SPAM | 0.0087 | — |
| L_amount_corrected | L_amount − SPAM | 0.0101 | — |
| L_optype_corrected | L_optype − SPAM | 0.0082 | — |
| L_append_corrected | L_append − SPAM | 0.0112 | — |
| L_replay_corrected | L_replay − SPAM | 0.0002 | — |

In-situ SPAM job (separate): Q_A p01 = 0.0024, Q_P p01 = 0.0000 (both ≤ 0.02). SPAM drift vs Arm 8 idle = 0.0000 (≤ 0.005). Gate passed.

## Decision against preregistered rule (Fields 20–22)

| PASS condition | Requirement | Observed | Met? |
| --- | --- | --- | --- |
| Destination changed | L_dest_corrected ≤ 0.02 | 0.0087 | ✅ |
| Amount changed | L_amount_corrected ≤ 0.02 | 0.0101 | ✅ |
| Operation changed | L_optype_corrected ≤ 0.02 | 0.0082 | ✅ |
| Extra action appended | L_append_corrected ≤ 0.02 | 0.0112 | ✅ |
| Approval replayed | L_replay_corrected ≤ 0.02 | 0.0002 | ✅ |
| Reverification recovery | S_reverified ≥ 0.90 | 0.9781 | ✅ |
| Boundary discrimination | Δ_B ≥ 0.70 | 0.9696 | ✅ |
| In-situ SPAM | ≤ 0.02 on both qubits | 0.0024 / 0.0000 | ✅ |
| Calibration drift | ≤ 0.005 | 0.0000 | ✅ |

→ PASS. Every class of post-approval alteration and the replayed approval fail closed; a re-verified mutation executes. See plots/arm_results.png and plots/integrity_discrimination.png.

## Secondary hypotheses (Field 19, descriptive — non-gating)

- H2a (alterations fail closed at the SPAM floor): point estimates read False for the four physical alteration arms. L_dest/L_amount/L_optype/L_append sit at ≈0.8–1.1%, a small residual above the pure-idle SPAM floor (0.00%) — consistent with the extra mid-circuit reset + re-measurement operations these arms carry, not with payload leakage. L_replay (0.02%) is statistically at the floor. All five remain far below the 0.02 ceiling, so the verdict is unaffected; the arms fail closed but are not indistinguishable from a bare idle qubit.

- H2b (reverification concordance): S_reverified (0.9781) and S_match (0.9808) — 95% Wilson intervals overlap → fresh re-verification of a mutated action restores execution within the matched-reference interval. Consistent with H2b.

- H2c (replay ≈ physical alteration): L_replay (0.0002) 99% interval does not overlap the four alteration arms — the replayed approval actually leaks less than the physically altered arms (it carries fewer error-accruing operations). Both mechanisms fail closed; they are not numerically identical on this hardware.

## Interpretation boundaries (Field 23)

This is a metrological characterization of a tamper-evident decision-to-execution binding on this qubit pair (Q_A=5, Q_P=6) on ibm_marrakesh at this calibration (2026-07-16). It is NOT new physics and NOT a cryptographic integrity guarantee. The “action signature” is abstracted to a single committed bit realized via a two-phase (approve → fresh re-verify) dynamic circuit; the alteration arms are representative encodings of alteration classes unified by the commitment mismatch they induce. “Leakage” means unauthorized payload activation (payload firing when the executed action does not match the approved action), not computational-basis leakage. A PASS demonstrates that, on this hardware and binding, post-approval alterations fail closed and a re-verified action executes; it does not establish security against an adversary, generalize to other bindings/qubits/backends, or assert MAC/signature strength. Findings do not generalize without replication.

## Relationship to prior work (Field 24)

ARK-441 (ibm_kingston) established the SPAM-resolved verify-then-execute (VBE) authorization boundary; ARK-446 replicated it cross-device on ibm_marrakesh; ARK-442 characterized its degradation under verification-to-execution delay (all PASS). ARK-444 extends the boundary from whether an authorization is valid to whether the executed action is exactly the approved action, binding the payload to a fresh execution-time verification and testing five post-approval alteration classes plus a reverification-recovery arm. The DENY-arm ceiling (L_corrected ≤ 0.02), the reverification floor (S_reverified ≥ 0.90), and the discrimination floor (Δ_B ≥ 0.70) carry over from ARK-441/442 and are all met here.

ARK-444 Results — Remnant Fieldworks Inc. / Derek Hone — 2026-07-16. Verdict PASS. Metrological characterization of a tamper-evident decision-to-execution binding; not new physics, not a cryptographic integrity guarantee. No Rescue After Failure.